Gantry Team: 9/29/26

Your list

- CORE: Routing/Movement, Safety, Attachments

- IMPORTANT: Interface (app/website), Simulation, Emergency Actions/Stops

- LATER: Extra attachment templates, Saved positions, Multiple paths/instructions in a row, Full-scale model

My suggestions: If you disagree with any of these, convince me!

CORE

- Routing/Movement: know where the hook is when the system starts, then move to a target inside the workspace.

- Safety: stay inside the workspace, don’t lift more than the limit, and include the emergency stop here. After a stop, the system should find its position again before moving.

- Attachments: one attachment for now (the hook) that can lift, lower, and check the weight.

IMPORTANT

- Interface (app/website). For Version 0, a simple way to send a command is enough.

LATER

- No changes.

Simulation

I don’t think this belongs in IMPORTANT. You need it first, to test your CORE before the hardware is ready. Start with dummy data. No real data is needed.

For example, hard-code two spots: A is at (2, 3) and B is at (8, 7). To move from A to B, the system just uses the straight-line (Euclidean) distance:

distance = √((8 − 2)² + (7 − 3)²) = √52 ≈ 7.2

Where does this data come from? For now, you write it yourself, in a small file or directly in your test. You don’t need the app for this. Later, the app sends the same data, and your CORE doesn’t change. The app is just a nicer way to enter it.

The same way, you can set the weight on the hook to any number, like 3 kg, to test the weight limit.

A question for you

I’m not sure about this one: is there one motor on each side of the gantry, or one motor driving both sides? How does it work in your design?

Related work: how does your lead do it?

Start from the lead you already chose (the paper or app). For each CORE item , answer:

- What does it do? Does your lead have something like this?

- How does it do it?

- What’s different for us? What will you follow, and what will you do differently?

If your lead doesn’t cover a CORE item, find one other source that does and answer the same questions. Search with the item plus your product, for example “CNC homing limit switch” or “overhead crane emergency stop.”